BZB-2U Test Sample Release Notes

Foreword

The BZB-2U is a main board updated product from the existing BZB-2. The major change is to replace the Parallel pot with one USB device port at the main board. It means the existing functions and specifications are not changed from the BZB-2 unless the communication port. The USB port is compatible to the USB2.0 full speed standard.

Implemented items and limitations

Hardware 
Mainboard updated, Power board integrated into the new mainboard.

Firmware

- Move out the parallel port functions and add the USB port for PnP printer functions. The application software must support the data flow control function for the USB port communication.

- Add the watch dog function to reset firmware if firmware trip into a dead loop.

- Add a dump printing mode to print out the raw data which received from the USB port. This mode may helpfully for engineer for the system development and after service. Continue press feed button will skip out the dump mode and print out the Demo page.

- The printheader heating level is adjusted to as like previous BZB-2 as possible.

Software 
BZB QLabel changed. For now, use the LPT2 port for the USB port printing. The USB port is support the data flow control function already.

Under con-structure items

- BZB QLabel: updated for the newer Microsoft operation system XP, Vista, Win7, Server 2003, Server 2008.

- DLL: to add EZPL-1 line mode commands process functions.

- Driver: to add EZPL-1 line mode commands and add the USB ID for BZB-2U.

- Mechanical: to make up the rear panel for the useless holes.

- Certification: to apply for CE EMI

- Product CD: to update the manuals, driver, DLL and BZB QLabel.